QUANTUM-539

Issue summary:

Very slow performance writing data from SNSM to WAS (Lattus-M).

Detailed description:

Writing to data to Lattus was very slow. The files were small. The issue was seen in two different sites. Customer was writing data over HTTPS

In line customer test:

In my testing I run 100,000 files at a time varying in size from 20 KB to 20 MB. I've tried many tests with different settings:

- Authorization Enabled / Disabled.

- Namespace Encrypted / Not Encrypted

- 12 Streams per controller / 48 streams per controller

- 20/4 policy / 18/11 policy

- HTTPS to controller / HTTP to controller

Customer complaint:

Why the write performance is so slow?

Components affected:

DSS clientdaemons, and storagedaemons

Pound

Disks

Tools and log files used to identify issue:

a- Clientdaemon logs were analyzed for put_duration statistics and any error messages.

there were objects written in slow times up to 22s on average, this was root caused to some slow disks the clientdaemon was failing to write to.

Since they used a policy that is as wide as the number of storagenodes performance can be easily affected by one bad nodes, or a few slow disks.

The affected disks were off lined, and performance was enhanced a bit, but not significantly. But yet was stable.

since the client used HTTPs we set the pound logs to debug and noticed that there every request takes two seconds.

5- How was it resolved?

After correlating the pound logs with the clientdaemon logs, seemed like there was a delay between pound and the clientdaemon, due to authentication and compression

Escalated to engineering which found that there is an issue with the pound version and issued a fix.